- PART ONE – GENERAL

  - SUMMARY

    - Work included: Furnishing and installing factory fabricated metal roof walkway.

    - Related Work:

      - Section 05 5136: Metal Catwalk & Walkways

      - Section 05 5213: Pipe and Tube Railings

      - Section 13 120: Pre-Engineered Structures

      - [Insert additional applicable specifications sections]

  - SUBMITTALS

    - Design: Architect, Contractor or Metal Building Manufacturer to submit proposed layout to Roof Walkway Manufacturer. Typical system mill finish - submit custom color requirements.

    - Product Data: Provide manufacturer’s product data for all materials in this specification (Bill or Materials).

    - Shop Drawings: Show profiles, accessories, location, and dimensions.

    - Samples: Manufacturer to provide upon request; sized to represent material adequately.

    - Load: Structural Engineer to be consulted to confirm load bearing / safety requirements are met.

    - Engineering: Load tests may be provided as an additional service.

    - Contract Closeout: Metal roof walkway manufacturer shall provide the manufacturer’s Warranty prior to the contract closeout.

  - PRODUCT HANDLING

    - All materials shall be delivered in the manufacturer’s original packaging.

    - Store materials in a dry, protected, well-vented area. The contractor shall thoroughly inspect the product upon receipt and report damaged material immediately to the delivering carrier and note such damage on the carrier’s freight bill of lading.

    - Remove protective wrapping immediately after installation [if applicable].

  - SUBSTITUTIONS

    - Proposals for substitution products shall be accepted only from bidding contractors and not less than (10) working days before the bid due date. The contractor guarantees that the proposed substitution shall meet the performance and quality standards of this specification.

  - JOB CONDITIONS

    - Verify that other trades with related work are complete before installing metal roof walkway.

    - Mounting surfaces shall be straight and secure; substrates shall be of proper width.

    - Refer to the construction documents, shop drawings, and manufacturer’s installation instructions.

    - Coordinate installation with roof panel manufacturer’s instructions before starting.

    - Observe all appropriate OSHA safety guidelines for this work.

  - WARRANTY/GUARANTEE

    - Manufacturer’s standard warranty: Materials shall be free of defects in material and workmanship for a period of one year from the date of purchase. Should a part fail to function in normal use within this period, the manufacturer shall furnish a new part at no charge.

- PART TW0 – PRODUCTS

  - MANUFACTURER

    - GrateSafe® Metal Roof Walkway - R & S Manufacturing and Sales Company, Inc., 630 Todd Lane, Unit B, Santa Paula, CA 93060. P. 1-805-375-3980 | F: 1-805-375-3981 | W: www.RSRoofProducts.com

  - GRATESAFE® METAL ROOF WALKWAY

    - Furnish and install metal roof walkways as indicated on plans.

    - Planking: Planks shall be anti-skid design 12” wide by 2-1/2” high and 10’ long (standard). Custom length planks are available up to 12’ in length. Flange options include male/male or female/male to enable interlocking assemblies of 24” widths increasing in 12” increments. Standard is mill finish G90 galvanized steel in 18 Gauge thickness.

    - Pitch-Correction: Optional pitch correction for a level surface up to a 4:12 slope for perpendicular to seam runs. Parallel to seam runs slope with the roof.

    - Accessories: Include standard 16-14 Gauge G90 galvanized steel supports, splice channels, and ledger angles where specified.

    - Hardware: Include clips, hold-down clamps, and fasteners where required.

    - S-5 Clamps 6061-T6 aluminum with stainless steel set screws, bolt and washer. Non-penetrating attachment on any Standing Seam Metal Roof Systems.

  - HAND RAILING

    - Safety Rail: Comply with OSHA Standard 29 CFR 1910.23. Designed to bear 200LB concentrated load applied to top rail at any point or 50lb/ft (whichever is greater). Schedule 40 Galvanized Handrail Piping. Nominal pipe size 1.66” O.D. and 0.140” nominal wall thickness. Optional pipe size of 1.90” O.D. Standard pipe length is 21’ for top and mid rail, and 42” for vertical posts.

    - Pipe Fittings: Hollaender Speed-Rail® aluminum-magnesium alloy 535 fittings manufactured to the requirements of ASTMB26-98. Hex set screws firmly lock the pipe into the fitting. Friction fit couplers are used to splice pipe lengths together.

  - FINISH OPTIONS

    - Standard is mill finish.

    - Powder coat finish.

    - Kynar finish.

- PART THREE – EXECUTION

  - INSPECTION

    - Verify that metal roof walkway installation will not disrupt other trades. Verify the substrate is dry, clean, and free of foreign matter. Report and correct defects prior to installation.

  - INSTALLAION

    - Submit product design drawings for review and approval to the architect or specifier before fabrication.

    - The installer shall check as-built conditions and verify the manufacturer’s metal roof walkway details for accuracy to fit the application prior to fabrication. The installer shall comply with the metal roof walkway Manufacturer's installation instructions.

    - The installer shall furnish mechanical fasteners consistent with the roof requirements.